Below is a copy of one my previous messages to forum members regarding the knee-wall; repair topic.

Hope the info is useful to each of you.

RB, it would be great if you could get the info stored in the files section.

Gene

From: Eugene Hoover [mailto:hoover@myglnc.com] 
Sent: Tuesday, June 07, 2011 4:42 PM
To: 'bmfern@telus.net'
Subject: FW: Pin Deflection and Frame Cracking:

Gerald I am forwarding content of a message I have shared with other members on the forum.  Lots of info, I hope it helps.

See the info below.  It contains a file and a link to other pictures in our Photo Gallery on the Titanium Owner Group Site. Feel free to contact me if there are additional questions.

Received your request for the file related to pin box deflection.  The file is attached, by way of background, I copied the pictures in that file from the Photo Gallery on the TOG User Group.  The original pictures were posted by TitaniumNick01 some time ago.  So I do not take credit for his work, only the fact that I created a PDF containing them.

Also provided below is a link to the TOG User Group Photo Gallery that points to annotated pictures that I created during my repair of our knee-wall issue.  As an update, we now have over 12,000 miles on our repair and pin deflection is still ¼” under static load.  So we are very happy with our approach.

http://www.titaniumrvowners.com/photos/thumbnails.php?album=364

Also, below is an excerpt from a message I sent to another TOG member regarding some additional reinforcement that I did:

“….Regards the aluminum angle; it was not across the top of the knee-wall,, but rather along the bottom edge of the bedroom floor fore and aft on the outside edge curb and highway sides and overlapping the existing aluminum angle.  It ran from the nose cap back to the turn down of the knee-wall portion of the existing side-wall framing.  One lip of the angle was mated and screwed to the existing angle and the other resting under the bedroom floor.  The new mating screws were driven into new holes and into the bedroom floor on the horizontal.  Screws were also ran up vertically from the bottom into the bedroom floor.

I did use a piece of flat stock 1/8th thick aluminum 2” wide and 8’ long across the top of the knee-wall junction, inside the bedroom and under the mattress.  Through that I ran the top knee-wall bolts, using fender washers both ends of the bolts.  Felt that would serve to better tie the top of the knee-wall structure to the rest of the unit…”

In response to another member’s question regarding tools used to separate the lower nose cover from the sidewalls and nose cap see the following:

See the attached picture of the tools I used to separate the pin box/knee-wall cover from the sidewalls.

You are correct, in that Glendale used some type of cement to bond the cover to the sidewalls.  They also used screws that will be found under the putty strip that goes around the perimeter of the cover.  Regards the putty strip across the lower nose; I did not fully remove the rubber gasket, but rather popped it out of the putty track groove and the “duct” taped it slightly forward of the cover seam.

After all of the putty tracks are removed around the perimeter of the cover, using a plastic scraper (not pictured) scrape off the residual putty and you may find several screws driven through the cover into the sidewalls.  The putty track screws across the underside of the nose generally go through both the cover and the nose lip, so no extra screws there.  Regards the screws, I replaced most of mine, since they were showing some corrosion.

The tools pictured consist of 2- 1” gasket scrapers (from Advance Auto), 2- 6” metal putty knifes, 1- 2” metal putty knife, and 1- 10” metal trowel all from Lowes. And, for driving, a rubber mallet.

I started with the 1” gasket scrapers and the 10” trowel.  I inserted the 10” trowel in the groove between the sidewall and the cover starting at the front (nose end).  I did not drive it in, but rather used it as a backer to protect the decals and sidewall finish.  I also used a piece of cardboard behind the scrapers.  It was about 18” square.  The mallet will leave a scuff mark if you miss the scraper’s handle and hit the side of the Titanium.   I then drove the 1” gasket scraper down in between the cover and sidewall (now protected by the 10” trowel.  Ever so gently I used the length of the scraper to pry the cover slightly away from the side wall.  I then drove one of the 6” blades down into the area between the cover and sidewall, its length was about the same as the depth of the lip between the cover and the sidewall.  I repeated that process walking the 10” blade and scrapers and putty knives along the edge of the cover.  Repeated the process on the other side in the same way.

I did have to replace several screws as I progressed after noting the rear was beginning to drop down before I was ready to complete the removal.

Removing the cover is a 2 person job.  BTW I did not have to remove the pin box.  Drop the rear-end down 8” or so, and then slide the cover to the rear enough to allow the cover’s front edge to clear the lip at the nose.  Then, lower the nose portion of the cover and take it all forward.  Installation is just the reverse.

On the go-back” used DAP Fiberglass Shower Surround Cement (Comes in a caulking gun tube). The warning on the package said it might be a little “hot”, i.e., burn through the fiberglass, but I believe the Titanium’s fiberglass is of a heavier thickness.  Just before re-installation, I applied it to the inside perimeter of the cover on the area touching the sidewalls only, that way the cement was spread down along the inside lip of the cover and not squished up along the sidewall where it would be visible.  There was no cement on the portion across the front of the cover.

I also purchased butyl putty tape from my local RV shop and after cleaning all of the putty tracks, used all new putty on the seams.  If you go that route, just be sure to watch the temperature of the putty tape.  If it gets too warm it tends to be a little sticky and hard to manage.  I kept my supply in my garage refrigerator until ready to use.

Still only getting ¼” movement after 12,000+ miles.  Made our round trip up to the Calgary Rally earlier this year after doing the repair. Happy camper here.

Good luck in the options you choose in getting your issue fixed.

Gene.

Cell – 252-315-3115.

Ps, be sure to read the several posts I generated after reporting our fix; use the search function in the Forum.  The bedroom floor under the mattress and where it connects to the side walls should be inspected when the lower nose cap is removed.